# CHAPTER 1

## 1.1 INTRODUCTION

The ESP8266 Local Messenger System is an innovative IoT-based project that provides a self-contained wireless communication platform without depending on the internet. Built on the ESP8266 microcontroller, the system acts as a Wi-Fi access point and hosts a web-based chat application, enabling users to send and receive messages in real time within a localized network.

Unlike conventional messaging applications that rely on external infrastructure, this system offers a standalone solution suitable for environments with limited, unreliable, or restricted internet access. It demonstrates the power of embedded systems by integrating networking, persistent storage, and hardware control in a single low-cost device.

The system uses an embedded web server running on the ESP8266 to deliver an intuitive HTML/JavaScript-based interface, ensuring cross-platform compatibility on smartphones, tablets, and laptops. Messages are stored in EEPROM using a circular buffer mechanism, ensuring persistence even after power cycles. The project also includes hardware control features, allowing users to toggle an LED remotely through the same interface, showcasing real-world IoT applications.

By combining client-server interaction, persistent storage, and embedded web technologies, the project not only provides a practical communication tool but also serves as an educational platform for students and developers to understand the fundamentals of IoT, networking, and microcontroller programming.

## 1.2 OBJECTIVES

The primary objectives of the Sorting Algorithm Visualization System are:

- Ensure Secure Communication: Apply WPA2 encryption and input validation to protect data from unauthorized access or misuse.

- Support Multi-User Connectivity: Enable multiple devices (4–8) to connect simultaneously and exchange messages reliably.

- Optimize System Performance:  Maintain low response time (<500ms) and efficient memory usage using a circular buffer.

- Demonstrate Hardware-Software Integration: Showcase LED control alongside messaging to highlight IoT functionality.

- Provide Cross-Platform Accessibility: Ensure compatibility across Android, iOS, Windows, and macOS devices.

- Enable Real-Time Interaction: Implement periodic auto-refresh and asynchronous requests for near real-time updates.

- Maintain Portability and Scalability: Design the system to be compact, low-power, and capable of scaling with multiple ESP8266 units.

- Offer Administrative Features: Allow bulk deletion of messages, system reset, and monitoring of active users and status.

- Promote Educational Value: Serve as a learning platform for students to understand IoT, networking, and embedded systems.

- Lay Foundation for Future Enhancements: Provide an open-source, modular design to allow extension into mesh networking, file transfer, or sensor integration.

## 1.3 FEASIBILITY STUDY

Technical Feasibility

- Technology Used: ESP8266 microcontroller, Arduino IDE/PlatformIO, HTML/CSS/JavaScript, and EEPROM.

- Reason: These technologies are open-source, lightweight, and widely supported.

- Compatibility: Runs on any device with Wi-Fi and a browser (smartphones, laptops, tablets).

- Extendibility: Easy to add advanced features such as mesh networking, file sharing, or sensor integration.

Operational Feasibility

- Ease of Use: Simple web interface accessible without installing any apps.

- Offline Functionality: Works even without internet connectivity, making it highly reliable.

- Maintenance: Low maintenance due to absence of external servers or databases.

- User Adaptability: Suitable for students, teachers, field researchers, and emergency responders.

Economic Feasibility

- Hardware Cost: ESP8266 module costs between $3–8 USD; supporting components $2–5 USD.

- Development Cost: Requires only free/open-source tools (Arduino IDE, libraries).

- Infrastructure Cost: No external server or internet cost; can run completely offline.

# CHAPTER 2

# 2.1 TECHNICAL REQUIREMENT

- Language: HTML/CSS/JS/C++

- Tools:  Browser.

- Operating system: Windows 11

- Minimum hardware requirements:8Gb RAM and 256 Gb SSD/HDD

## 2.2 SYSTEM ANALYSIS

Problem Definition

In many situations, internet connectivity may be unavailable, unreliable, or restricted. Traditional messaging applications become unusable under such conditions. There is a need for a self-contained local communication system that can provide real-time group messaging and basic IoT device control without relying on external infrastructure.

The ESP8266 Local Messenger addresses this by creating its own Wi-Fi network and hosting a web-based chat platform, allowing users to connect, communicate, and control hardware devices locally.

Objectives of the System

- Develop a standalone messaging system independent of internet infrastructure.

- Implement persistent storage for messages and LED state using EEPROM.

- Provide a responsive, browser-based interface accessible from multiple devices simultaneously.

- Demonstrate IoT principles through integration of hardware (LED control) and software.

- Ensure low-cost, low-power, and portable operation for educational and practical use.

- Support multi-user connectivity and near real-time communication.

System Requirements Analysis

Functional Requirements

- ESP8266 should create a Wi-Fi Access Point with WPA2 security.

- Embedded web server must serve an interface for messaging and LED control.

- Users should be able to send and receive messages in real-time.

- Messages must be stored in EEPROM for persistence across power cycles.

- System should allow LED ON/OFF control through the same interface.

Non-Functional Requirements

- Should support at least 4–8 concurrent users.

- Response time must be under 500ms for message delivery.

- System should remain stable during 24-hour continuous operation.

- Must be platform-independent (work on Android, iOS, Windows, macOS).

- Should have low power consumption (<100mA typical)

System Architecture

- Hardware Components:

- ESP8266 microcontroller (NodeMCU/Wemos/ESP-01)

- LED connected to GPIO14 for hardware control

- Standard 3.3V power supply

- Software Components:

- Wi-Fi Access Point Manager – Handles network creation and client connections

- Web Server – Serves UI and API endpoints (/send, /messages, /led, /clear)

- Message Management System – Stores/retrieves messages with a circular buffer in EEPROM

- EEPROM Interface – Maintains data persistence across power cycles

- Hardware Control Interface – Manages LED state and updates EEPROM

# CHAPTER 3

## 3.1 FLOWCHART

# CHAPTER 4

## 4.1 CODE

#include <ESP8266WiFi.h>

#include <ESP8266WebServer.h>

#include <EEPROM.h>

// Configuration

const char* AP_SSID = "DCN";

const char* AP_PASSWORD = "12345678";

const int MAX_MESSAGES = 50;

const int MESSAGE_LENGTH = 200;

const int USERNAME_LENGTH = 32;

const int EEPROM_SIZE = 4096;

const int LED_PIN = 14; // Built-in LED pin (can be changed to any GPIO pin)

// Web server

ESP8266WebServer server(80);

// Message structure (updated to include username)

struct Message {

char content[MESSAGE_LENGTH];

char username[USERNAME_LENGTH];

unsigned long timestamp;

bool valid;

};

// Global variables

Message messages[MAX_MESSAGES];

int messageCount = 0;

int currentIndex = 0;

bool ledState = false;

void setup() {

Serial.begin(115200);

delay(1000);

// Initialize LED pin

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW); // Turn off LED initially

// Initialize EEPROM

EEPROM.begin(EEPROM_SIZE);

// Load messages from EEPROM

loadMessages();

// Setup WiFi Access Point

WiFi.mode(WIFI_AP);

WiFi.softAP(AP_SSID, AP_PASSWORD);

Serial.println("Access Point Started");

html += "<div class='controls'>";

html += "<button onclick='refreshMessages()' class='btn-secondary'>Refresh</button>";

html += "<button onclick='toggleLED()' class='btn-led' id='ledButton'>LED OFF</button>";

html += "<button onclick='clearMessages()' class='btn-danger'>Clear All</button>";

html += "</div>";

html += "</div>";

html += "</div>";

html += "</div>";

html += "<script src='/script.js'></script>";

html += "</body>";

html += "</html>";

server.send(200, "text/html", html);

}

void handleSendMessage() {

if (!server.hasArg("message") || !server.hasArg("username")) {

server.send(400, "application/json", "{\"error\":\"Message and username required\"}");

return;

}

String message = server.arg("message");

String username = server.arg("username");

message.trim();

username.trim();

if (message.length() == 0 || username.length() == 0) {

server.send(400, "application/json", "{\"error\":\"Message and username cannot be empty\"}");

return;

}

addMessage(message, username);

server.send(200, "application/json", "{\"success\":true}");

}

void handleGetMessages() {

String json = getMessagesJSON();

server.send(200, "application/json", json);

}

void handleClearMessages() {

clearAllMessages();

server.send(200, "application/json", "{\"success\":true}");

}

void handleLED() {

if (!server.hasArg("state")) {

server.send(400, "application/json", "{\"error\":\"LED state required\"}");

return;

}

String state = server.arg("state");

ledState = (state == "on" || state == "1" || state == "true");

// Apply LED state (normal logic: HIGH = ON, LOW = OFF)

digitalWrite(LED_PIN, ledState ? HIGH : LOW);

// Save LED state to EEPROM

saveMessages();

String response = "{\"success\":true,\"state\":\"" + String(ledState ? "on" : "off") + "\"}";

server.send(200, "application/json", response);

}

void handleLEDStatus() {

String response = "{\"state\":\"" + String(ledState ? "on" : "off") + "\"}";

server.send(200, "application/json", response);

}

void handleCSS() {

String css = "* { margin: 0; padding: 0; box-sizing: border-box; }";

css += "body { font-family: -apple-system, BlinkMacSystemFont, 'Segoe UI', Roboto, sans-serif; background: linear-gradient(135deg, #667eea 0%, #764ba2 100%); min-height: 100vh; padding: 10px; }";

css += ".container { max-width: 800px; margin: 0 auto; background: white; border-radius: 15px; box-shadow: 0 10px 30px rgba(0,0,0,0.2); overflow: hidden; height: calc(100vh - 20px); display: flex; flex-direction: column; }";

css += "h1 { background: linear-gradient(45deg, #667eea, #764ba2); color: white; text-align: center; padding: 20px; margin: 0; font-size: 1.5em; }";

css += ".status { display: flex; justify-content: space-between; padding: 15px 20px; background: #f8f9fa; border-bottom: 1px solid #e9ecef; font-size: 0.9em; color: #6c757d; }";

css += ".user-setup { flex: 1; display: flex; align-items: center; justify-content: center; padding: 40px; }";

css += ".user-input { display: flex; gap: 10px; width: 100%; max-width: 400px; }";

css += ".chat-area { flex: 1; display: flex; flex-direction: column; }";

css += ".current-user { display: flex; justify-content: space-between; align-items: center; padding: 10px 20px; background: #e8f5e8; border-bottom: 1px solid #d4edda; font-size: 0.9em; }";

css += ".messages { flex: 1; overflow-y: auto; padding: 20px; background: #f8f9fa; }";

css += ".message { background: white; margin-bottom: 15px; padding: 15px; border-radius: 10px; box-shadow: 0 2px 5px rgba(0,0,0,0.1); animation: fadeIn 0.3s ease-out; }";

css += ".message-header { display: flex; justify-content: space-between; align-items: center; margin-bottom: 8px; }";

css += ".message-username { font-weight: bold; color: #667eea; font-size: 0.9em; }";

css += ".message-time { font-size: 0.8em; color: #6c757d; }";

css += ".message-content { line-height: 1.4; word-wrap: break-word; }";

css += ".loading { text-align: center; color: #6c757d; padding: 20px; }";

css += ".input-area { background: white; padding: 20px; border-top: 1px solid #e9ecef; }";

css += ".input-group { display: flex; margin-bottom: 15px; }";

css += "#messageInput, #usernameInput { flex: 1; padding: 12px; border: 2px solid #e9ecef; border-radius: 8px 0 0 8px; font-size: 16px; outline: none; transition: border-color 0.3s; }";

css += "#messageInput:focus, #usernameInput:focus { border-color: #667eea; }";

css += "button { padding: 12px 20px; background: linear-gradient(45deg, #667eea, #764ba2); color: white; border: none; border-radius: 0 8px 8px 0; cursor: pointer; font-size: 16px; transition: opacity 0.3s; }";

css += "button:hover { opacity: 0.9; }";

css += ".controls { display: flex; gap: 10px; flex-wrap: wrap; }";

css += ".btn-secondary { background: linear-gradient(45deg, #6c757d, #495057) !important; border-radius: 8px !important; flex: 1; min-width: 80px; }";

css += ".btn-danger { background: linear-gradient(45deg, #dc3545, #c82333) !important; border-radius: 8px !important; flex: 1; min-width: 80px; }";

css += ".btn-led { background: linear-gradient(45deg, #28a745, #20c997) !important; border-radius: 8px !important; flex: 1; min-width: 80px; }";

css += ".btn-change-user { background: linear-gradient(45deg, #6c757d, #495057) !important; border-radius: 6px !important; padding: 6px 12px !important; font-size: 0.8em !important; white-space: nowrap; }";

css += ".btn-led.led-on { background: linear-gradient(45deg, #ffc107, #fd7e14) !important; }";

css += "@keyframes fadeIn { from { opacity: 0; transform: translateY(10px); } to { opacity: 1; transform: translateY(0); } }";

css += "@media (max-width: 600px) { body { padding: 5px; } .container { height: calc(100vh - 10px); } h1 { font-size: 1.3em; padding: 15px; } .status { flex-direction: column; gap: 5px; text-align: center; } .status span { margin: 2px 0; } .controls { flex-direction: column; gap: 8px; } .controls button { min-width: unset; } .current-user { padding: 8px 15px; font-size: 0.8em; } .btn-change-user { padding: 4px 8px !important; font-size: 0.7em !important; } .input-group { margin-bottom: 10px; } .input-area { padding: 15px; } }";

css += "@media (max-width: 480px) { .status { font-size: 0.8em; } .controls { gap: 6px; } .controls button { padding: 10px 8px; font-size: 14px; } h1 { font-size: 1.2em; padding: 12px; } }";

css += ".no-messages { text-align: center; color: #6c757d; padding: 40px 20px; font-style: italic; }";

server.send(200, "text/css", css);

}

void handleJS() {

String js = "let messageCount = 0;";

js += "let currentUsername = '';";

js += "let ledState = false;";

js += "function formatTime(timestamp) {";

js += "const now = Date.now();";

js += "const bootTime = now - performance.now();";

js += "const messageTime = bootTime + timestamp;";

js += "const diffMs = now - messageTime;";

js += "const diffMins = Math.floor(diffMs / 60000);";

js += "const diffHours = Math.floor(diffMins / 60);";

js += "const diffDays = Math.floor(diffHours / 24);";

js += "if (diffDays > 0) return diffDays + 'd ago';";

js += "if (diffHours > 0) return diffHours + 'h ago';";

js += "if (diffMins > 0) return diffMins + 'm ago';";

js += "return 'now';";

js += "}";

js += "function setUsername() {";

js += "const input = document.getElementById('usernameInput');";

js += "const username = input.value.trim();";

js += "if (username === '') {";

js += "alert('Please enter a username');";

js += "return;";

js += "}";

js += "currentUsername = username;";

js += "document.getElementById('currentUser').textContent = username;";

js += "document.getElementById('userSetup').style.display = 'none';";

js += "document.getElementById('chatArea').style.display = 'flex';";

js += "refreshMessages();";

js += "document.getElementById('messageInput').focus();";

js += "}";

js += "function changeUsername() {";

js += "document.getElementById('userSetup').style.display = 'flex';";

js += "document.getElementById('chatArea').style.display = 'none';";

js += "document.getElementById('usernameInput').value = '';";

js += "document.getElementById('usernameInput').focus();";

js += "}";

js += "function displayMessages(messages) {";

js += "const container = document.getElementById('messages');";

js += "if (messages.length === 0) {";

js += "container.innerHTML = '<div class=\"no-messages\">No messages yet. Be the first to send one!</div>';";

js += "return;";

js += "}";

js += "let html = '';";

js += "messages.forEach(msg => {";

js += "}";

js += "});";

js += "document.getElementById('usernameInput').addEventListener('keypress', function(e) {";

js += "if (e.key === 'Enter') {";

js += "setUsername();";

js += "}";

js += "});";

js += "document.addEventListener('DOMContentLoaded', function() {";

js += "refreshLEDStatus();";

js += "setInterval(function() {";

js += "if (document.getElementById('chatArea').style.display !== 'none') {";

js += "refreshMessages();";

js += "}";

js += "refreshLEDStatus();";

js += "}, 5000);";

js += "document.getElementById('usernameInput').focus();";

js += "});";

server.send(200, "application/javascript", js);

}

void handleNotFound() {

server.send(404, "text/plain", "Not found");

}

## 4.2 OUTPUT

ESP8266 Board

Entering User Name

Chat History and LED on mode

LED off mode

## 4.3 TESTING

Functionality Testing

  - Multi-user Messaging: Successful communication between 4 concurrent users.

  - Message Persistence: Data retention verified across multiple power cycles.

  - Interface Responsiveness: Cross-platform compatibility confirmed on Android, iOS, Windows, and macOS.

  - Edge Case Handling: Proper behaviour with maximum message length and storage capacity.

Performance Testing

  - Network Stability: Continuous 24-hours operation without connectivity issues.

  - Memory Management: Efficient EEPROM usage with no memory leaks detected.

  - Response Time: Average message delivery time under 500ms.

  - Concurrent Load: Stable operation with 6 simultaneous active users.

Security Testing

  - Input Validation: XSS and injection attack prevention verified.

  - Network Security: WPA2 encryption functionality confirmed.

  - Error Handling: Graceful handling of malformed request and network errors.

CHAPTER 5

## 5.1 MERITS

- Offline Communication:  Works without internet or external servers, ensuring reliable communication in remote or restricted areas.

- Low-Cost Solution: Built using the ESP8266 microcontroller, which is inexpensive and widely available, making the project cost-effective.

- Portability: Compact design allows easy deployment anywhere, powered by USB or a small battery.

- Persistence Data Storage: Messages and LED state are stored in EEPROM, ensuring data is retained across power cycles.

- User Friendly Interface: Simple, responsive web-based interface accessible from any device with a Wi-Fi connection and browser.

- Cross-platform Capability: Works seamlessly across Android, iOS, Windows, and macOS without needing any dedicated application.

- Educational Value: Serves as a hands-on project to learn concepts of embedded systems, networking, IoT, and web development.

## DE-MERITS

- Limited Wi-Fi Range: Coverage is restricted to approximately 50–100 meters, depending on obstacles and environment.

- Restricted Message Capacity: Can store only up to 50 messages due to EEPROM size; older messages are automatically deleted.

- Basic Hardware Integration: Current system only demonstrates LED control; lacks advanced IoT features like sensors, actuators, or multiple device controls.

- Performance Constraints: With limited processing power and memory, ESP8266 cannot support heavy workloads or large-scale messaging like cloud-based systems.

## 5.3 CONCLUSION

The ESP8266 Local Messenger Project successfully demonstrates how a low-cost microcontroller can be used to create a self-contained communication system independent of internet infrastructure. By leveraging the ESP8266’s built-in Wi-Fi capabilities, the project establishes a secure local network where multiple users can connect, exchange messages, and control hardware devices through a simple, browser-based interface.

The use of EEPROM storage ensures that both messages and LED state persist across power cycles, adding reliability to the system. The project also highlights the importance of IoT integration, showcasing how embedded devices can combine software (web-based messaging) and hardware (LED control) to provide practical real-world applications.

Although the system has limitations such as restricted range, limited storage, and basic functionality compared to internet-based messaging platforms, it is highly effective in specific use cases like classrooms, laboratories, emergency communication, and remote locations where internet access is unavailable.

## 5.4 FUTURE ENHANCEMENT

- Mesh Networking: Connect multiple ESP8266 modules to form a mesh network, extending communication range and coverage area.

- User Authentication:  Implement login systems with unique usernames and passwords to prevent unauthorized access.

- File and Media Sharing: Extend the system to support transfer of images, documents      and audio files.

- Voice Messaging: Add functionality for recording and transmitting short voice clips, enhancing usability in emergencies.

BIBLIOGRAPHY

- www.geeksforgeeks.org

- www.codeproject.com

- www.tutorialspoint.com

- www.claude.com